	[image: image1.png]



	Technical Customer Information
	KI0301e0

	
	Bucher AG, Beringer Hydraulik Lift with DSV544*-LIFT 
"analog" process "Beringer"
	22.01.2003

	
	
	Page: 1 of 1



Parameter and Variables DSV 5445 - Lift, "analog" process "Beringer"

This is the description of the data from BERINGER.KOM or BERIPACK.KOM with default settings:
For initial operation with BERINGER.KOM there have only the 2 parameters "F0" and "t", according to  the operating instructions from Bucher AG, to be changed (adaption, type of motor and DSV5445).
The system BERIPAC with BERIPACK.KOM is preset completely. The values of the factory settings are in brackets, e.g. F0 (750). The small printed values are hidden parameters for experts only:


	Adr
	Parameter, variable
	Meaning
	Factory settings
	Range of values

	F0
	Rotor flow
	Motor rotor flux 
	500 (750)
	50...2000

	F1
	Rated speed
	Nominal speed according to rating plate
	2905 (1475)
	100...4000 1/min

	F2
	Synchr. speed
	Synchronous speed
	3000 (1500)
	100...4500 1/min

	F10
	2094024 / 0E4A
	512-encoder=4002, 1024-Geber=2001
	4002 (1000)
	2001...4002

	F23
	0E4A / 4
	512-encoder=128, 1024-Geber=256
	128 (512)
	256

	F29
	Version-flag
	old=0, new=255
	255
	0 or 255

	F31
	Number of poles
	number of motor poles
	2 (4)
	2...64

	i
	I-part run
	I-part speed controller
	300
	4...400

	k
	P-part run
	P-part speed controller
	300
	100...5000

	t
	Rotor time const.
	Motor rotor time constant 
	100
	25...1000

	0B34
	Version of progr. (>75)
	Program revision index (read only)
	>75
	read only

	0E00
	Stand. of speed "1"
	Evaluation of the analog setpoint 
(660 = 10 = 3000 1/min)
	660 (1452)
	-1500...1500

	0E0C
	Start hyst. "1"
	Hysteresis to hide drift 
(1 = 5mV)
	1
	0...10

	0E10
	Offset sym. "1"
	Offset to displace the setpoint voltage (1 = 5mV, -1 = -5mV)
	0
	-100...100

	0E26
	Off-decel. delay
	Delay controller switch-off
	1
	1...1000

	0E40
	Excess speed "A2"
	Speed threshold 1/min for output A2 (0E40 is also employed for the fixed speed using input E3 resp. E4; the ramps are stored in 0E64, analog in 0E62) 
	1500
	0...4000

	0E4A
	Encoder pulses (new)
	Encoder pulse number
	512 (2048)
	512, 1024, 2048

	0E58
	RS 485 "b5" A01...A99

new programs use f25
	Adressing, if setpoint group control is performed "digitally".
	1.00
	0...99

	0E60
	Service-flag (emergency operation without encoder)
	Emergency operation  without encoder
(set rotor flux F0 to maximum)
	0
	0/255

	0E62
	Ramp gradient 01
	Set ramp flat Y0/Y1
	335 (1000)
	1...5000

	0E64
	Ramp gradient 34
	Set ramp steep Y0/Y1
	67 (67)
	1...5000

	0E68
	Emergency-flag OFF = 0
	Flag for switching-off  the phase monitoring ("phase failure")
	  255
	0/255


	(
	The above named program versions are effective from january 2003.
For WinDietz there are the programs BERINGER.CNF, BERINGER.KOM, BERIPACK.KOM and BERINGER.UPD (update for existing older inverters) available from 13. Jan. 2003. 
The parameter F10, F23 and the variables 0E0C, 0E10, 0E58 are not shown.
This program version can also be used in older DSV5445. For this the experienced user has to set the parameter F29 to "0"; valid only for firmware TUDYxxx.
The correct function of the analog setpoint input is only guaranteed with our AddOn-filter. All measures for noise suppression have to be completed carefully.


